Raspberry pi obstacle detection robot

This project demonstrates how to build an obstacle-avoiding robot using a Raspberry Pi and an ultrasonic sensor (HC-SR04). The robot detects obstacles in its path and changes direction autonomously using simple logic programmed in Python.

Features

•	Obstacle Detection: Measures distance using an HC-SR04 ultrasonic sensor and avoids obstacles based on pre-defined thresholds.

•	Motor Control: Uses an L298N motor driver to control two DC motors for forward, backward, and turning movements.

•	Simple Wiring: Includes an easy-to-follow wiring diagram for seamless assembly.

How It Works

1.	The ultrasonic sensor continuously measures the distance to obstacles in front of the robot.

2.	If an obstacle is detected within a set threshold distance (e.g., 15 cm), the robot stops, moves backward briefly, and turns to avoid the obstacle.

3.	If no obstacles are detected, the robot continues to move forward.

Getting Started

Prerequisites

•	Install Raspbian OS on your Raspberry Pi.

•	Install the required Python libraries:

pip install RPi.GPIO

Setup

1.	Connect the components as per the wiring diagram provided in this repository.

2.	Copy the Python script to your Raspberry Pi.

3.	Run the script using:

robot.py

Bill of Materials in INR

| Component | Quantity | Estimated Cost (INR) | Notes |
| --- | --- | --- | --- |
| Raspberry Pi | 1 | 3,500–6,000 | Raspberry Pi 4 or Raspberry Pi Zero W with GPIO support. |
| Micro SD Card (16GB or higher) | 1 | 400–600 | For Raspberry Pi OS. |
| HC-SR04 Ultrasonic Sensor | 1 | 150–250 | For obstacle distance measurement. |
| L298N Motor Driver | 1 | 250–400 | To control the motors. |
| DC Motors (6V) | 2 | 300–500 | For robot movement (gear motors recommended). |
| Wheels | 2 | 150–300 | Attach to the DC motors. |
| Caster Wheel | 1 | 100–200 | For stability and balance. |
| Battery Pack (6-12V) | 1 | 500–800 | Power the motors (e.g., AA battery holder or Li-ion pack). |
| Connecting Wires | 1 Set | 200–300 | For connections (male-to-female, male-to-male). |

Total Estimated Cost: ₹5,550 – ₹9,350

This is an approximate cost based on market rates in India. Prices may vary depending on the vendor or region. Would you like recommendations for local or online stores?

Pin Diagram (Without Servo):

| Raspberry Pi GPIO | Component |
| --- | --- |
| 5V | VCC Pin of HC-SR04 |
| GPIO 5 | Trigger Pin of HC-SR04 |
| GPIO 6 | Echo Pin of HC-SR04 |
| Ground (GND) | GND Pin of HC-SR04 |
| GPIO 8 | Left Motor IN1 |
| GPIO 9 | Left Motor IN2 |
| GPIO 10 | Right Motor IN1 |
| GPIO 11 | Right Motor IN2 |

Powering the Motors and Ultrasonic Sensor:

1.	5V for Ultrasonic Sensor (HC-SR04):

•	Connect the VCC pin of the ultrasonic sensor to the Raspberry Pi’s 5V pin.

•	Connect the GND pin of the ultrasonic sensor to one of the Raspberry Pi’s GND pins.

2.	Motor Driver Power:

•	Connect the positive (+) terminal of an external battery to the motor driver’s VCC.

•	Connect the negative (-) terminal of the battery to the motor driver’s GND.

•	Connect the motor driver’s GND to the Raspberry Pi’s GND pin to establish a common ground between the battery and the Pi.

This setup will allow the ultrasonic sensor to detect obstacles, and the motor driver will control the motors based on the sensor data. Make sure the motors’ battery voltage is appropriate for the motor driver you’re using.

Python Code (Without Servo Motor)

import RPi.GPIO as GPIO

import time

# Set up GPIO mode

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

# Motor Pins

MLA = 8  # Left motor IN1

MLB = 9  # Left motor IN2

MRA = 10 # Right motor IN1

MRB = 11 # Right motor IN2

# Ultrasonic Sensor Pins

TRIG_PIN = 5

ECHO_PIN = 6

# Distance threshold for obstacle detection (in cm)

OBSTACLE_THRESHOLD = 15

# Set up motor pins as output

GPIO.setup(MLA, GPIO.OUT)

GPIO.setup(MLB, GPIO.OUT)

GPIO.setup(MRA, GPIO.OUT)

GPIO.setup(MRB, GPIO.OUT)

# Set up ultrasonic sensor pins

GPIO.setup(TRIG_PIN, GPIO.OUT)

GPIO.setup(ECHO_PIN, GPIO.IN)

# Motor control functions

def move_forward():

GPIO.output(MLA, GPIO.HIGH)

GPIO.output(MLB, GPIO.LOW)

GPIO.output(MRA, GPIO.HIGH)

GPIO.output(MRB, GPIO.LOW)

def move_backward():

GPIO.output(MLA, GPIO.LOW)

GPIO.output(MLB, GPIO.HIGH)

GPIO.output(MRA, GPIO.LOW)

GPIO.output(MRB, GPIO.HIGH)

def turn_right():

GPIO.output(MLA, GPIO.HIGH)

GPIO.output(MLB, GPIO.LOW)

GPIO.output(MRA, GPIO.LOW)

GPIO.output(MRB, GPIO.HIGH)

def turn_left():

GPIO.output(MLA, GPIO.LOW)

GPIO.output(MLB, GPIO.HIGH)

GPIO.output(MRA, GPIO.HIGH)

GPIO.output(MRB, GPIO.LOW)

def stop():

GPIO.output(MLA, GPIO.LOW)

GPIO.output(MLB, GPIO.LOW)

GPIO.output(MRA, GPIO.LOW)

GPIO.output(MRB, GPIO.LOW)

# Function to measure distance using the ultrasonic sensor

def measure_distance():

# Trigger the sensor

GPIO.output(TRIG_PIN, True)

time.sleep(0.00001)  # 10µs pulse

GPIO.output(TRIG_PIN, False)

# Measure the time for the echo to return

start_time = time.time()

stop_time = time.time()

# Save the start time

while GPIO.input(ECHO_PIN) == 0:

start_time = time.time()

# Save the arrival time

while GPIO.input(ECHO_PIN) == 1:

stop_time = time.time()

# Calculate the time difference

elapsed_time = stop_time - start_time

# Calculate distance (speed of sound is approximately 34300 cm/s)

distance = (elapsed_time * 34300) / 2

return distance

# Main loop

try:

while True:

distance = measure_distance()

print(f"Distance: {distance} cm")

if distance <= OBSTACLE_THRESHOLD:

# Obstacle detected, stop and take action

stop()

time.sleep(0.3)

move_backward()

time.sleep(0.2)

stop()

time.sleep(0.3)

# Check distances on the right and left by turning slightly

turn_right()

time.sleep(0.5)

distance_right = measure_distance()

stop()

time.sleep(0.3)

turn_left()

time.sleep(1)  # Longer turn to face left

distance_left = measure_distance()

stop()

time.sleep(0.3)

# Decide direction based on measured distances

if distance_right > distance_left:

turn_right()

else:

turn_left()

time.sleep(0.5)

stop()

else:

# No obstacle, move forward

move_forward()

time.sleep(0.1)

except KeyboardInterrupt:

print("Program stopped by User")

finally:

# Clean up GPIO pins on exit

GPIO.cleanup()

Code Explanation

1.	Motor Control Functions: move_forward, move_backward, turn_right, turn_left, and stop control the motors based on the GPIO output.

2.	Ultrasonic Sensor: measure_distance triggers the ultrasonic sensor and calculates the distance in centimeters.

3.	Main Loop:

•	Measures distance using the ultrasonic sensor.

•	If an obstacle is detected within the threshold (OBSTACLE_THRESHOLD), the robot stops, moves backward briefly, and then turns to the right or left depending on the distances measured on both sides.

•	If no obstacle is detected, the robot moves forward.

Important Notes:

•	Be sure to share a common ground between the motor power source and the Raspberry Pi for proper functionality.

•	Adjust the OBSTACLE_THRESHOLD variable if you want to detect obstacles at a different distance.

License

This project is licensed under the MIT License.

Contributions

Contributions are welcome! Feel free to fork this repository and create pull requests to improve the project.